Mini Project

Verilog & VHDL

UART Rx,Tx

SPI Rx,Tx

I2C RX,Tx

UART

Verilog Code

UART Tx

`timescale 1ns / 1ps

module Uart_Tx(

output reg tx,

output reg tx_done,

input clk,

input rst,

input tx_start,

input [7:0] data_in

);

parameter clk_freq = 1_000_000;

parameter baud_rate = 9600;

localparam bits_per_clk = clk_freq / baud_rate;

localparam

idle      = 3'd0,

start_bit = 3'd1,

data_bits = 3'd2,

parity_bit = 3'd3,

stop1     = 3'd4,

stop2     = 3'd5,

done      = 3'd6;

reg [2:0] state;

reg [7:0] clk_count;

reg [3:0] bit_index;

reg parity;

reg [7:0] data;

always @(posedge clk or posedge rst) begin

if (rst) begin

state <= idle;

clk_count <= 0;

tx <= 1;

tx_done <= 0;

bit_index <= 0;

end else begin

case (state)

idle: begin

tx <= 1;

tx_done <= 0;

if (tx_start) begin

data <= data_in;

parity <= ^data_in;

clk_count <= 0;

bit_index <= 0;

state <= start_bit;

end

end

start_bit: begin

tx <= 0;

clk_count <= clk_count + 1;

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

state <= data_bits;

end

end

data_bits: begin

tx <= data[bit_index];

clk_count <= clk_count + 1;

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

if (bit_index == 7)

state <= parity_bit;

else

bit_index <= bit_index + 1;

end

end

parity_bit: begin

tx <= parity;

clk_count <= clk_count + 1;

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

state <= stop1;

end

end

stop1: begin

tx <= 1;

clk_count <= clk_count + 1;

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

state <= stop2;

end

end

stop2: begin

tx <= 1;

clk_count <= clk_count + 1;

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

state <= done;

end

end

done: begin

tx_done <= 1;

state <= idle;

end

endcase

end

end

endmodule

UART Rx

`timescale 1ns / 1ps

module Uart_Rx(

output reg parity_err,

output reg rx_done,

input clk, rst, rx,

output reg [7:0] data_out

);

parameter clk_freq = 1_000_000;

parameter baud_rate = 9600;

localparam bits_per_clk = clk_freq / baud_rate;

localparam mid_sample = bits_per_clk / 2;

localparam

idle       = 3'd0,

start_bit  = 3'd1,

data_bits  = 3'd2,

parity_bit = 3'd3,

stop1      = 3'd4,

stop2      = 3'd5,

cleanup    = 3'd6;

reg [2:0] state;

reg [7:0] clk_count;

reg [3:0] bit_index;

reg [7:0] data;

reg parity;

always @(posedge clk or posedge rst) begin

if (rst) begin

clk_count <= 0;

bit_index <= 0;

rx_done <= 0;

parity_err <= 0;

state <= idle;

data <= 0;

data_out <= 0;

end else begin

case (state)

idle: begin

rx_done <= 0;

if (rx == 0) begin

clk_count <= 0;

state <= start_bit;

end

end

start_bit: begin

clk_count <= clk_count + 1;

if (clk_count == mid_sample) begin

if (rx == 0) begin

clk_count <= 0;

bit_index <= 0;

state <= data_bits;

end else begin

state <= idle;

end

end

end

data_bits: begin

clk_count <= clk_count + 1;

if (clk_count == mid_sample) begin

data[bit_index] <= rx;

end

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

if (bit_index == 7)

state <= parity_bit;

else

bit_index <= bit_index + 1;

end

end

parity_bit: begin

clk_count <= clk_count + 1;

if (clk_count == mid_sample)

parity <= rx;

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

state <= stop1;

end

end

stop1: begin

clk_count <= clk_count + 1;

if (clk_count == mid_sample && rx != 1) begin

state <= idle;  // Error

end

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

state <= stop2;

end

end

stop2: begin

clk_count <= clk_count + 1;

if (clk_count == mid_sample && rx != 1) begin

state <= idle;

end

if (clk_count == bits_per_clk - 1) begin

clk_count <= 0;

state <= cleanup;

end

end

cleanup: begin

rx_done <= 1;

parity_err <= (parity != ^data);

data_out <= data;

clk_count <= 0;

state <= idle;

end

endcase

$display("Time: %0t | State: %0d | RX: %b | BitIndex: %0d | clk_count: %0d | Data: %b | Done: %b",

$time, state, rx, bit_index, clk_count, data, rx_done);

end

end

endmodule

UART Top

`timescale 1ns / 1ps

module Uart_top(

input clk, rst,

input tx_start,

input [7:0] data_in,

output tx,

output [7:0] data_out,

output rx_done,

output parity_err

);

wire tx_line;

wire tx_done;

Uart_Tx tx_inst(

.tx(tx_line),

.tx_done(tx_done),

.clk(clk),

.rst(rst),

.tx_start(tx_start),

.data_in(data_in)

);

Uart_Rx rx_inst(

.parity_err(parity_err),

.rx_done(rx_done),

.clk(clk),

.rst(rst),

.rx(tx_line),

.data_out(data_out)

);

assign tx = tx_line;

endmodule

Test Bnech Code

`timescale 1ns / 1ps

module tb_uart;

reg clk, rst, tx_start;

reg [7:0] data_in;

wire tx;

wire [7:0] data_out;

wire rx_done, parity_err;

Uart_top uut (

.clk(clk),

.rst(rst),

.tx_start(tx_start),

.data_in(data_in),

.tx(tx),

.data_out(data_out),

.rx_done(rx_done),

.parity_err(parity_err)

);

initial clk = 0;

always #500 clk = ~clk;

initial begin

rst = 1;

tx_start = 0;

data_in = 8'b10101010;

#2000;

rst = 0;

#2000;

tx_start = 1;

#1000;

tx_start = 0;

wait(rx_done == 1);

$display("Received: %b (0x%h), Parity Error: %b", data_out, data_out, parity_err);

#10000;

$finish;

end

endmodule

VHDL Code

UART_Tx

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity UART_Tx is

Port (

clk, rst       : in  std_logic;

tx_start       : in  std_logic;

data_in        : in  std_logic_vector(7 downto 0);

tx             : out std_logic;

tx_done        : out std_logic

);

end UART_Tx;

architecture Behavioral of UART_Tx is

constant clk_freq     : integer := 1000000;

constant baud_rate    : integer := 9600;

constant bits_per_clk : integer := clk_freq / baud_rate;

type state_type is (idle, start, data_bit, parity_bit, stop1, stop2, cleanup);

signal state      : state_type := idle;

signal clk_count  : integer := 0;

signal bit_index  : integer := 0;

signal data_reg   : std_logic_vector(7 downto 0) := (others => '0');

signal parity     : std_logic := '0';

signal tx_reg     : std_logic := '1';

begin

tx <= tx_reg;

process(clk, rst)

begin

if rst = '1' then

state      <= idle;

tx_done    <= '0';

clk_count  <= 0;

tx_reg     <= '1';

bit_index  <= 0;

elsif rising_edge(clk) then

case state is

when idle =>

tx_done <= '0';

tx_reg  <= '1';

if tx_start = '1' then

data_reg <= data_in;

clk_count <= 0;

bit_index <= 0;

parity <= data_in(0) xor data_in(1) xor data_in(2) xor data_in(3) xor

data_in(4) xor data_in(5) xor data_in(6) xor data_in(7);

state <= start;

end if;

when start =>

tx_reg <= '0';

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

state <= data_bit;

else

clk_count <= clk_count + 1;

end if;

when data_bit =>

tx_reg <= data_reg(bit_index);

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

if bit_index = 7 then

state <= parity_bit;

else

bit_index <= bit_index + 1;

end if;

else

clk_count <= clk_count + 1;

end if;

when parity_bit =>

tx_reg <= parity;

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

state <= stop1;

else

clk_count <= clk_count + 1;

end if;

when stop1 =>

tx_reg <= '1';

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

state <= stop2;

else

clk_count <= clk_count + 1;

end if;

when stop2 =>

tx_reg <= '1';

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

state <= cleanup;

else

clk_count <= clk_count + 1;

end if;

when cleanup =>

tx_done <= '1';

tx_reg <= '1';

state <= idle;

clk_count <= 0;

end case;

end if;

end process;

end Behavioral;

UART Rx

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity UART_Rx is

Port (

clk, rst       : in  std_logic;

rx             : in  std_logic;

data_out       : out std_logic_vector(7 downto 0);

rx_done        : out std_logic;

parity_err     : out std_logic

);

end UART_Rx;

architecture Behavioral of UART_Rx is

constant clk_freq     : integer := 1000000;

constant baud_rate    : integer := 9600;

constant bits_per_clk : integer := clk_freq / baud_rate;

constant mid_sample   : integer := bits_per_clk / 2;

type state_type is (idle, start, data_bit, parity_bit, stop1, stop2, cleanup);

signal state      : state_type := idle;

signal clk_count  : integer := 0;

signal bit_index  : integer := 0;

signal data_reg   : std_logic_vector(7 downto 0) := (others => '0');

signal parity     : std_logic := '0';

begin

process(clk, rst)

begin

if rst = '1' then

clk_count  <= 0;

state      <= idle;

bit_index  <= 0;

data_out   <= (others => '0');

rx_done    <= '0';

parity_err <= '0';

elsif rising_edge(clk) then

case state is

when idle =>

rx_done <= '0';

if rx = '0' then

clk_count <= 0;

state <= start;

end if;

when start =>

clk_count <= clk_count + 1;

if clk_count = mid_sample then

if rx = '0' then

clk_count <= 0;

bit_index <= 0;

state <= data_bit;

else

state <= idle;

end if;

end if;

when data_bit =>

clk_count <= clk_count + 1;

if clk_count = mid_sample then

data_reg(bit_index) <= rx;

end if;

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

if bit_index = 7 then

state <= parity_bit;

else

bit_index <= bit_index + 1;

end if;

end if;

when parity_bit =>

clk_count <= clk_count + 1;

if clk_count = mid_sample then

parity <= rx;

end if;

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

state <= stop1;

end if;

when stop1 =>

clk_count <= clk_count + 1;

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

state <= stop2;

end if;

when stop2 =>

clk_count <= clk_count + 1;

if clk_count = bits_per_clk - 1 then

clk_count <= 0;

state <= cleanup;

end if;

when cleanup =>

data_out   <= data_reg;

rx_done    <= '1';

parity_err <= parity xor (data_reg(0) xor data_reg(1) xor data_reg(2) xor data_reg(3) xor

data_reg(4) xor data_reg(5) xor data_reg(6) xor data_reg(7));

state <= idle;

clk_count <= 0;

end case;

end if;

end process;

end Behavioral;

UART Top

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity UART_Top is

Port (clk,rst,tx_start:in std_logic;data_in: in std_logic_vector(7 downto 0);data_out: out std_logic_vector(7 downto 0);tx_done,rx_done,parity_err,tx:out std_logic );

end UART_Top;

architecture Behavioral of UART_Top is

signal tx_line :std_logic;

begin

UART_Tx_inst: entity work.UART_Tx port map(

clk => clk,

rst => rst,

tx_start=>tx_start,

data_in=>data_in,

tx=>tx_line,

tx_done=>tx_done);

UART_RX_inst: entity work.UART_Rx port map(

clk => clk,

rst => rst,

data_out=>data_out,

rx=>tx_line,

rx_done=>rx_done,

parity_err=>parity_err);

tx<=tx_line;

end Behavioral;

Test Bench Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_UART is

end tb_UART;

architecture Behavioral of tb_UART is

signal clk,rst,tx_start: std_logic;

signal data_in:  std_logic_vector(7 downto 0);

signal data_out: std_logic_vector(7 downto 0);

signal tx_done,rx_done,parity_err,tx: std_logic;

constant clk_period : time := 1 us;

begin

uut:entity work.UART_Top port map (

clk=>clk,

rst=>rst,

tx_start=>tx_start,

data_in=>data_in,

tx_done=>tx_done,

data_out=>data_out,

rx_done=>rx_done,

parity_err=>parity_err,

tx=>tx);

clock : process begin

clk<='0';wait for 5ns;

clk<='1';wait for 5ns;

end process;

stim_proc: process

begin

rst <= '1';

tx_start <= '0';

data_in <= (others => '0');

wait for 5 * clk_period;

rst <= '0';

wait for clk_period;

data_in <= "10101010";

tx_start <= '1';

wait for clk_period;

tx_start <= '0';

wait until rx_done = '1';

wait for 10 * clk_period;

assert false report "Simulation complete" severity note;

wait;

end process;

end Behavioral;

SPI

Verilog Code

`timescale 1ns / 1ps

module SPI_Verilog(

input clk,rst,start,miso,

input [7:0]data_in,

output reg ss,reg_done,mosi,sck,

output reg [7:0]data_rec

);

parameter clk_div=4;

reg [1:0]state;

reg [2:0]bit_count;

reg [7:0]shift_reg;

reg [clk_div-1:0]clk_div_count;

parameter

idle =2'd0,

load = 2'd1,

transfer =2'd2,

done = 2'd3;

always @(posedge clk or posedge rst) begin

if (rst) begin

state<=idle;

sck<=0;

clk_div_count<=0;

bit_count<=0;

shift_reg<=0;

mosi<=0;

data_rec<=8'b0;

ss<=1;

reg_done<=0;end

else begin

case (state)

idle: begin

if (start==0) begin

sck<=0;

ss<=1;

reg_done<=0;

end

else begin

state<=load;end

end

load: begin

shift_reg<=data_in;

bit_count<=3'b111;

ss<=0;

clk_div_count<=0;

state<=transfer;

end

transfer: begin

clk_div_count<=clk_div_count+1;

if (clk_div_count==(clk_div/2)-1) begin

sck<=1;

mosi<=shift_reg[bit_count];

end

if (clk_div_count==clk_div-1) begin

sck<=0;

data_rec[bit_count]<=miso;

clk_div_count<=0;

if (bit_count==0) begin

state<=done;

end

else begin

bit_count<=bit_count-1;

end

end

end

done: begin

sck<=0;

reg_done<=1;

ss<=1;

state<=idle;

end

endcase

end

end

endmodule

Test Bench Code

`timescale 1ns / 1ps

module tb_SPI;

reg clk,rst,start,miso;

reg [7:0]data_in;

wire ss,reg_done,mosi,sck;

wire [7:0]data_rec ;

SPI_Verilog uut(

.clk(clk),

.rst(rst),

.start(start),

.miso(miso),

.data_in(data_in),

.ss(ss),

.reg_done(reg_done),

.mosi(mosi),

.sck(sck),

.data_rec(data_rec));

initial begin

clk=0;

forever #5 clk=~clk;

end

initial begin

rst=1;

start=0;

data_in=8'b10101010;

miso=0;

#20;

rst=0;

#20;

start=1;

#10;

start=0;

wait (ss==0);

#10; miso = 1;

#40; miso = 0;

#40; miso = 1;

#40; miso = 0;

#40; miso = 1;

#40; miso = 0;

#40; miso = 1;

#40; miso = 0;

wait(reg_done);

$display("SPI Transfer Complete");

$display("Received data: %b", data_rec);

#20;

$finish;

end

endmodule

VHDL Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity SPI_VHDL is

Port (clk,rst,start,miso:in std_logic;data_in:in std_logic_vector(7 downto 0);data_rec:out std_logic_vector(7 downto 0);ss,sck,mosi,reg_done:out std_logic);

end SPI_VHDL;

architecture Behavioral of SPI_VHDL is

type state_type is(idle,load,transfer,done);

signal state:state_type:=idle;

constant clk_div:integer :=4;

signal bit_count:integer;

signal shift_reg: std_logic_vector(7 downto 0):=(others =>'0');

signal clk_div_cnt:integer;

signal rec_reg:  std_logic_vector(7 downto 0):=(others =>'0');

signal ss_int: std_logic:='1';

signal mosi_int:std_logic:='0';

signal done_int:std_logic:='0';

signal sck_int:std_logic:='0';

begin

sck<=sck_int;

reg_done<=done_int;

ss<=ss_int;

mosi<=mosi_int;

data_rec<=rec_reg;

process (clk,rst) begin

if(rst='1') then

done_int<='0';

ss_int<='1';

mosi_int<='0';

done_int<='0';

sck_int<='0';

bit_count<=0;

shift_reg<=(others=>'0');

rec_reg<=(others=>'0');

clk_div_cnt<=0;

elsif (rising_edge(clk)) then

case state is

when idle =>

sck_int<='0';

done_int<='0';

ss_int<='1';

if (start='1') then

state<=load;

end if;

when load =>

ss_int<='0';

shift_reg<=data_in;

bit_count<=7;

clk_div_cnt<=0;

state<=transfer;

when transfer =>

clk_div_cnt <= clk_div_cnt + 1;

if (clk_div_cnt=(clk_div/2)-1) then

sck_int<='1';

mosi_int<=shift_reg(bit_count);

end if;

if (clk_div_cnt=clk_div-1) then

sck_int<='0';

rec_reg(bit_count)<=miso;

sck_int<='0';

if (bit_count=0) then

state<=done;

else

bit_count<=bit_count-1;

end if ;

end if;

when done  =>

sck_int<='0';

ss_int<='1';

done_int<='1';

state<=idle;

end case;

end if ;

end process;

end Behavioral;

Test Bench Code

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_SPI is

end tb_SPI;

architecture Behavioral of tb_SPI is

component SPI_VHDL

Port (

clk       : in  std_logic;

rst       : in  std_logic;

start     : in  std_logic;

miso      : in  std_logic;

data_in   : in  std_logic_vector(7 downto 0);

ss        : out std_logic;

reg_done  : out std_logic;

mosi      : out std_logic;

sck       : out std_logic;

data_rec  : out std_logic_vector(7 downto 0)

);

end component;

signal clk,rst,start,miso: std_logic;

signal data_in: std_logic_vector(7 downto 0):= x"AA" ;

signal data_rec: std_logic_vector(7 downto 0);

signal ss,sck,mosi,reg_done: std_logic;

begin

uut: entity work.SPI_VHDL port map(

clk=>clk,

rst=>rst,

start=>start,

miso=>miso,

data_in=>data_in,

data_rec=>data_rec,

ss=>ss,

sck=>sck,

mosi=>mosi,

reg_done=>reg_done);

clock: process begin

clk<='0'; wait for 5ns;

clk<='1'; wait for 5ns;

end process;

stim : process

begin

start <= '0';

miso <= '0';

rst <= '1';

wait for 10 ns;

rst <= '0';

wait for 10 ns;

start <= '1';

wait for 10 ns;

start <= '0';

wait until ss = '0';

wait for 10 ns; miso <= '1';

wait for 40 ns; miso <= '0';

wait for 40 ns; miso <= '1';

wait for 40 ns; miso <= '0';

wait for 40 ns; miso <= '1';

wait for 40 ns; miso <= '0';

wait for 40 ns; miso <= '1';

wait for 40 ns; miso <= '0';

wait until reg_done = '1';

report "SPI Transfer Complete";

wait for 50 ns;

assert false report "End of simulation." severity failure;

end process;

end Behavioral;

I2C

Verilog Code

I2c Master

module i2c_master #(

parameter clk_freq = 50000000,

parameter i2c_freq = 100000

)

(

input wire clk,

input wire rst,

input wire start,

input wire rw,

input wire [6:0] addr7,

input wire [7:0] reg_addr,

input wire [7:0] data_in,

input wire do_read,

output reg busy,

output reg ack_err,

output reg arb_lost,

output reg[7:0] read_data,

output reg done,

inout wire sda,

inout wire scl

);

localparam integer clk_per_half_scl = clk_freq/(i2c_freq*2);

localparam integer cntr_w = $clog2(clk_per_half_scl+1);

reg sda_oe;

reg scl_oe;

assign sda = sda_oe ? 1'b0 : 1'bz;

assign scl = scl_oe ? 1'b0 : 1'bz;

wire sda_in = sda;

wire scl_in = scl;

localparam

idle= 4'd0,

start_bit = 4'd1,

send_addr = 4'd2,

wait_ack1 = 4'd3,

send_reg = 4'd4,

wait_ack2 = 4'd5,

write_data = 4'd6,

wait_ack3 = 4'd7,

rep_start = 4'd8,

send_addr_rd = 4'd9,

wait_ack4 = 4'd10,

read_data_bit = 4'd11,

send_nack = 4'd12,

stop = 4'd13,

done_bit = 4'd14;

reg [3:0] state,next_state;

reg [cntr_w-1:0] half_scl_cnt;

reg scl_edge;

reg phase;

reg [7:0] tx_shift;

reg [2:0] bit_index;

reg expecting_ack;

always @(posedge clk or negedge rst) begin

if (!rst)

arb_lost <= 1'b0;

else if (sda_oe && sda_in)

arb_lost <= 1'b1;

else

arb_lost <= arb_lost;

end

always @(posedge clk or negedge rst) begin

if (!rst) begin

half_scl_cnt <= 1'b0;

scl_edge <= 1'b0;

phase <= 1'b0;

end

else begin

if (half_scl_cnt == (clk_per_half_scl-1)) begin

half_scl_cnt <= 1'b0;

scl_edge <= 1'b1;

phase <= ~phase;

end

else begin

half_scl_cnt <= half_scl_cnt+1;

scl_edge <= 1'b0;

end

end

end

always @(posedge clk or negedge rst) begin

if (!rst) begin

state <= idle;

busy <= 1'b0;

ack_err <= 1'b0;

done <= 1'b0;

read_data <= 8'h00;

sda_oe <= 1'b0;

scl_oe <= 1'b0;

bit_index <= 3'd7;

tx_shift <= 8'h00;

expecting_ack <= 1'b0;

end

else begin

done <= 1'b0;

case (state)

idle : begin

ack_err <= 1'b0;

arb_lost <= 1'b0;

if (start) begin

busy <= 1'b1;

state <= start_bit;

end

end

start_bit : begin

scl_oe <= 1'b0;

if (scl_in && phase == 1'b0) begin

sda_oe <= 1'b1;

end

if (scl_edge && phase == 1'b1) begin

state <= send_addr;

tx_shift <= {addr7,rw};

bit_index <= 3'd7;

expecting_ack <= 1'b0;

scl_oe <= 1'b1;

half_scl_cnt <= 1'b0;

phase <= 1'b0;

end

end

send_addr : begin

if (scl_edge && phase == 1'b0) begin

sda_oe <= ~tx_shift[bit_index];

scl_oe <= 1'b0;

end

if (scl_edge && phase == 1'b1) begin

scl_oe <= 1'b1;

if (bit_index == 1'b0) begin

state <= wait_ack1 ;

sda_oe <= 1'b0;

expecting_ack <= 1'b1;

end

else begin

bit_index <= bit_index-1;

end

end

end

wait_ack1 : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

end

if (scl_edge && phase == 1'b1) begin

if (sda_in == 1'b1) begin

ack_err <= 1'b1;

scl_oe <= 1'b1;

bit_index <= 3'd7;

tx_shift <= reg_addr;

expecting_ack <= 1'b0;

end

state <= send_reg;

end

end

send_reg : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

sda_oe <= ~tx_shift[bit_index];

end

if (scl_edge && phase == 1'b1) begin

scl_oe <= 1'b1;

if (bit_index == 1'b0) begin

state <= wait_ack2;

sda_oe <= 1'b0;

expecting_ack <= 1'b1;

end

else begin

bit_index <= bit_index-1;

end

end

end

wait_ack2 : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

end

if (scl_edge && phase == 1'b1) begin

if (sda_in == 1'b1) begin

ack_err <= 1'b1;

end

scl_oe <= 1'b1;

if (do_read) begin

state <= rep_start;

end

else begin

state <= write_data;

tx_shift <= data_in;

bit_index <= 3'd7;

end

end

end

write_data : begin

if (scl_edge && phase == 1'b0) begin

sda_oe <= ~tx_shift[bit_index];

scl_oe <= 1'b0;

end

if (scl_edge && phase == 1'b1) begin

scl_oe <= 1'b1;

if (bit_index == 1'b0) begin

state <= wait_ack3;

sda_oe <= 1'b0;

end

else begin

bit_index <= bit_index-1;

end

end

end

wait_ack3 : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

end

if (scl_edge && phase == 1'b1) begin

if (sda_in == 1'b1) begin

ack_err <= 1'b1;

scl_oe <= 1'b1;

state <= stop;

end

end

end

rep_start : begin

scl_oe <= 1'b0;

sda_oe <= 1'b0;

if (scl_in) begin

sda_oe <= 1'b1;

tx_shift <= {addr7,1'b1};

bit_index <= 3'd7;

sda_oe <= 1'b1;

scl_oe <= 1'b1;

state <= send_addr_rd;

end

end

send_addr_rd : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

sda_oe <= ~tx_shift[bit_index];

end

if (scl_edge && phase == 1'b1) begin

scl_oe <= 1'b1;

if (bit_index == 1'b0) begin

state <= wait_ack4;

sda_oe <= 1'b0;

end

else begin

bit_index <= bit_index-1;

end

end

end

wait_ack4 : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

end

if (scl_edge && phase == 1'b1) begin

if (sda_in == 1'b1) begin

ack_err <= 1'b1;

scl_oe <= 1'b1;

bit_index <= 3'd7;

sda_oe <= 1'b0;

end

state <= read_data_bit;

end

end

read_data_bit : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

end;

if (scl_edge && phase == 1'b1) begin

read_data[bit_index] <= sda_in;

scl_oe <= 1'b1;

if (bit_index == 1'b0) begin

state <= send_nack;

end

else begin

bit_index <= bit_index-1;

end

end

end

send_nack : begin

if (scl_edge && phase == 1'b0) begin

scl_oe <= 1'b0;

sda_oe <= 1'b0;

end

if (scl_edge && phase == 1'b1) begin

scl_oe <= 1'b1;

state <= stop;

end

end

stop : begin

scl_oe <= 1'b0;

if (scl_in) begin

sda_oe <= 1'b0;

state <= done_bit;

end

else if (phase ==1'b0)

sda_oe <=1'b1;

end

done_bit : begin

busy <= 1'b0;

done <= 1'b1;

read_data <= data_in;

state <= idle;

end

default : begin

state <= idle;

end

endcase

end

end

endmodule

i2c Slave

module i2c_slave #(

parameter slave_addr = 7'h42

)(

input wire clk,rst,

inout wire sda,

inout wire scl,

input wire sim_clk_str

);

reg sda_oe;

reg scl_oe;

assign sda = sda_oe ? 1'b0 : 1'bz ;

assign scl = scl_oe ? 1'b0 : 1'bz ;

wire sda_in = sda;

wire scl_in = scl;

localparam

idle = 4'd0,

addr= 4'd1,

ack_addr = 4'd2,

reg_bit = 4'd3,

ack_reg = 4'd4,

data = 4'd5,

ack_data = 4'd6,

read_byte = 4'd7,

ack_read = 4'd8;

reg [3:0] state;

reg [7:0] shift;

reg [2:0] bit_cnt;

reg rw_flag;

reg [7:0] reg_file [0:15];

reg [3:0] reg_ptr;

reg expecting_reg;

reg sda_prev,scl_prev;

wire start_det = (sda_in == 1'b1 && sda_in == 1'b0 && scl_in == 1'b1);

wire stop_det = (sda_in == 1'b0 && sda_in == 1'b1 && scl_in == 1'b1);

integer i;

initial begin

for (i= 0 ; i < 16 ; i=i+1 )

reg_file[i]=i;

state = idle;

sda_oe = 1'b0;

scl_oe = 1'b0;

sda_prev = 1'b1;

scl_prev = 1'b1;

bit_cnt = 3'd7;

shift = 8'b0;

reg_ptr = 4'b0;

//expecting_ack = 1'b0;

end

always @(posedge clk or negedge rst) begin

if (!rst) begin

state = idle;

sda_oe = 1'b0;

scl_oe = 1'b0;

sda_prev = 1'b1;

scl_prev = 1'b1;

bit_cnt = 3'd7;

shift = 8'b0;

reg_ptr = 4'b0;

//expecting_ack = 1'b0;

end

else begin

sda_prev <= sda_in;

scl_prev <= scl_in;

end

if (start_det) begin

state <= addr;

bit_cnt <= 3'd7;

shift <= 8'b0;

scl_oe <= 1'b0;

end

else if (stop_det) begin

state <= idle;

sda_oe <= 1'b0;

scl_oe <= 1'b0;

end

else begin

if (scl_prev == 1'b0 && scl_in == 1'b1) begin

case(state)

addr: begin

shift[bit_cnt] <= sda_in;

if (bit_cnt == 1'b0) begin

if (shift[7:1] == slave_addr) begin

state <= ack_addr;

rw_flag <= shift[0];

end

else begin

state <= idle;

end

bit_cnt <= 3'd7;

end

else begin

bit_cnt <= bit_cnt - 1;

end

end

ack_addr : begin

sda_oe <= 1'b1;

state <= reg_bit;

end

reg_bit : begin

sda_oe <= 1'b0;

bit_cnt <= 3'd7;

if (bit_cnt == 1'b0) begin

reg_ptr <= shift[7:0];

bit_cnt <= 3'd7;

if (rw_flag == 1'b0) begin

state <= ack_reg;

expecting_reg <= 1'b1;

end

else begin

state <= ack_reg;

end

end

else begin

bit_cnt <= bit_cnt-1;

end

end

ack_reg : begin

sda_oe <= 1'b1;

if (rw_flag) begin

sda_oe <= 1'b0;

state <= read_byte;

bit_cnt <= 3'd7;

shift <= reg_file[reg_ptr];

end

else begin

sda_oe <= 1'b0;

state <= data;

bit_cnt <= 3'd7;

shift <= 8'h00;

end

end

data : begin

if (bit_cnt == 1'b0) begin

reg_file[reg_ptr] <= shift;

state <= ack_data;

bit_cnt <= 3'd7;

end

else begin

bit_cnt <= bit_cnt-1;

end

end

ack_data : begin

sda_oe <= 1'b1;

sda_oe <= 1'b0;

state <= idle;

end

read_byte : begin

sda_oe <= ~shift[bit_cnt];

if (bit_cnt == 1'b0) begin

bit_cnt <= 3'd7;

sda_oe <= 1'b0;

state <= idle;

end

else begin

bit_cnt <= bit_cnt-1;

end

end

default : begin

state <= idle;

end

endcase

end

end

end

endmodule

i2c Top

module i2c_top();

wire sda;

wire scl;

pullup pu_sda(sda);

pullup pu_scl(scl);

reg clk,rst;

initial begin

clk = 1'b0;

forever #5

clk = ~clk;

end

reg start;

reg rw;

reg [6:0] addr7;

reg [7:0] reg_addr;

reg [7:0] data_in;

reg do_read;

wire busy;

wire ack_err;

wire arb_lost;

wire[7:0] read_data;

wire done;

i2c_master #(.clk_freq(100000000),.i2c_freq(100000)) master_inst(

.clk(clk),

.rst(rst),

.start(start),

.rw(rw),

.addr7(addr7),

.reg_addr(reg_addr),

.data_in(data_in),

.do_read(do_read),

.busy(busy),

.ack_err(ack_err),

.arb_lost(arb_lost),

.read_data(read_data),

.done(done),

.sda(sda),

.scl(scl)

);

reg sim_clk_str;

i2c_slave #(.slave_addr(7'h42)) slave_inst(

.clk(clk),

.rst(rst),

.scl(scl),

.sda(sda),

.sim_clk_str(sim_clk_str)

);

initial begin

rst = 0;

start = 0;

rw = 0;

addr7 = 7'h42;

reg_addr = 8'h01;

data_in = 8'hA5;

do_read = 0;

sim_clk_str= 0;

#100;

rst= 1;

end

endmodule

i2c TestBench

module i2c_tb();

wire sda;

wire scl;

reg clk,rst;

pullup(sda);

pullup(scl);

initial begin

clk= 1'b0;

forever #5

clk <= ~clk;

end

reg start;

reg rw;

reg [6:0] addr7;

reg [7:0] reg_addr;

reg [7:0] data_in;

reg do_read;

wire busy;

wire ack_err;

wire arb_lost;

wire[7:0] read_data;

wire done;

i2c_master #(.clk_freq(100000000),.i2c_freq(100000)) master_inst(

.clk(clk),

.rst(rst),

.start(start),

.rw(rw),

.addr7(addr7),

.reg_addr(reg_addr),

.data_in(data_in),

.do_read(do_read),

.busy(busy),

.ack_err(ack_err),

.arb_lost(arb_lost),

.read_data(read_data),

.done(done),

.sda(sda),

.scl(scl)

);

reg sim_clk_str;

i2c_slave #(.slave_addr(7'h42)) slave_inst(

.clk(clk),

.rst(rst),

.scl(scl),

.sda(sda),

.sim_clk_str(sim_clk_str)

);

initial begin

$dumpfile("dump.vcd");

$dumpvars(0,i2c_tb);

rst = 1'b0;

start = 1'b0;

sim_clk_str =1'b0;

addr7 = 8'h42;

reg_addr = 8'h02;

data_in = 8'h5a;

do_read = 1'b0;

#200;

rst = 1'b1;

#200;

$display("TEST1: Write 0x%02x to reg 0x%02x", data_in, reg_addr);

start = 1;

rw = 0;

do_read = 0;

#10 start = 0;

wait (done);

#200;

$display("DONE write: ack_err=%b arb_lost=%b", ack_err, arb_lost);

$display("TEST2: Read from reg 0x%02x", reg_addr);

start = 1;

rw = 1;

do_read = 1;

#10 start = 0;

wait (done);

#100;

$display("Read data = 0x%02x ack_err=%b", read_data, ack_err);

$display("TEST3: Clock stretching simulated");

sim_clk_str = 1;

data_in = 8'hCC;

reg_addr = 8'h03;

start= 1;

rw = 0;

do_read = 0;

#10 start = 0;

wait (done);

sim_clk_str = 0;

#100;

$display("After stretch write ack_err=%b", ack_err);

#500;

$finish;

end

endmodule

VHDL Code

I2c Master

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity i2c_master is

generic(

clk_freq : integer := 50000000;

i2c_freq : integer := 100000

);

port(

clk,rst : in std_logic;

start : in std_logic;

rw : in std_logic;

addr7 : in std_logic_vector(6 downto 0);

reg_addr : in std_logic_vector(7 downto 0);

data_in : in std_logic_vector(7 downto 0);

busy : out std_logic;

ack_err : out std_logic;

done : out std_logic;

read_data : out std_logic_vector(7 downto 0);

sda : inout std_logic;

scl : inout std_logic

);

end i2c_master;

architecture behavioral of i2c_master is

constant clk_per_half : integer := clk_freq/(i2c_freq*2);

type state_t is(

idle,start_bit,

send_addr,ack1,

send_reg, ack2,

send_data, ack3,

stop_bit, done_bit

);

signal state : state_t := idle;

signal half_cnt : integer range 0 to clk_per_half := 0;

signal scl_tick : std_logic := '0';

signal phase : std_logic := '0';

signal sda_oe : std_logic := '0';

signal scl_oe : std_logic := '0';

signal tx_shift : std_logic_vector(7 downto 0);

signal bit_count : integer range 0 to 7 := 7 ;

signal sda_in : std_logic;

signal scl_in : std_logic;

begin

sda <= '0' when sda_oe = '1' else 'Z';

scl <= '0' when scl_oe = '1' else 'Z';

sda_in <= sda;

scl_in <= scl;

process(clk,rst)

begin

if (rst = '0') then

half_cnt <= 0;

scl_tick <= '0';

phase <= '0';

elsif rising_edge(clk) then

if half_cnt = clk_per_half-1 then

half_cnt <= 0;

scl_tick <= '1';

phase <= not phase;

else

half_cnt <= half_cnt+1;

scl_tick <= '0';

end if;

end if;

end process;

process (clk,rst)

begin

if rst = '0' then

state <= idle;

busy <= '0';

ack_err <= '0';

done <= '0';

sda_oe <= '0';

scl_oe <= '0';

bit_count <= 7;

read_data <= (others => '0');

elsif rising_edge(clk) then

done <= '0';

case state is

when idle =>

busy <= '0';

ack_err <= '0';

if (start='1') then

busy <= '1';

state <= start_bit;

end if;

when start_bit =>

scl_oe <= '0';

if (phase = '0') then

sda_oe <= '1';

end if;

if (scl_tick = '1' and phase = '1') then

scl_oe <= '1';

tx_shift <= addr7 & rw;

bit_count <= 7;

state <= send_addr;

end if;

when send_addr =>

if scl_tick = '1' and phase = '0' then

sda_oe <= not tx_shift(bit_count);

scl_oe <= '0';

elsif scl_tick = '1' and phase = '1' then

scl_oe <= '1';

if bit_count = 0 then

sda_oe <= '0';

state <= ack1;

else

bit_count <= bit_count-1;

end if;

end if;

when ack1 =>

if scl_tick = '1' and phase = '1' then

if sda_in = '1' then

ack_err <= '1';

end if;

tx_shift <= reg_addr;

bit_count <= 7;

state <= send_reg;

end if;

when send_reg =>

if scl_tick = '1' and phase = '0' then

sda_oe <= not tx_shift(bit_count);

scl_oe <= '0';

elsif scl_tick = '1' and phase = '1' then

scl_oe <= '1';

if bit_count = 0 then

sda_oe <= '0';

state <= ack2;

else

bit_count <= bit_count-1;

end if;

end if;

when ack2 =>

if scl_tick = '1' and phase = '1' then

if sda_in = '1' then

ack_err <= '1';

end if;

tx_shift <= data_in;

bit_count <= 7;

state <=send_data;

end if;

when send_data =>

if scl_tick = '1' and phase = '0' then

sda_oe <= not tx_shift(bit_count);

scl_oe <= '0';

elsif scl_tick = '1' and phase = '1' then

scl_oe <= '1';

if bit_count = 0 then

sda_oe <= '0';

state <= ack3;

else

bit_count <= bit_count-1;

end if;

end if;

when ack3 =>

if scl_tick='1' and phase = '1' then

if sda_in = '1' then

ack_err <= '1';

end if;

state <= stop_bit;

end if;

when done_bit =>

busy <= '0';

done <= '1';

state <= idle;

when stop_bit =>

if scl_tick = '1' and phase = '0' then

scl_oe <= '0';  -- release SCL

sda_oe <= '1';  -- keep SDA low

elsif scl_tick = '1' and phase = '1' then

sda_oe <= '0';  -- SDA goes high while SCL high = STOP

state <= done_bit;

end if;

end case;

end if;

end process;

end behavioral;

i2c Slave

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity i2c_slave is

generic (

slave_addr : std_logic_vector(6 downto 0) := "1000010"

);

port(

clk,rst : in std_logic;

sda : inout std_logic;

scl : inout std_logic

);

end i2c_slave;

architecture behavioral of i2c_slave is

type state_t is (

idle,

addr,

addr_ack,

reg_addr,

reg_ack,

write_data,

write_ack,

read_data,

read_ack

);

signal state : state_t := idle;

signal sda_oe : std_logic := '0';

signal sda_in : std_logic ;

signal bit_count : integer range 0 to 7 := 7;

signal rx_shift : std_logic_vector( 7 downto 0) := (others => '0');

signal tx_shift : std_logic_vector(7 downto 0) := (others => '0');

signal rw : std_logic := '0';

signal reg_ptr : integer range 0 to 255 := 0;

type mem_t is array ( 0 to 255) of std_logic_vector(7 downto 0);

signal mem : mem_t := (others =>(others => '0'));

signal sda_d,scl_d : std_logic;

begin

sda <= '0' when sda_oe = '1' else 'Z';

sda_in <= sda;

process (clk)

begin

if rising_edge (clk) then

sda_d <= sda;

scl_d <= scl;

end if;

end process;

process (clk,rst)

begin

if (rst = '0') then

state <= idle;

sda_oe <= '0';

bit_count <= 7;

elsif rising_edge(clk) then

if (sda_d = '1' and sda = '0' and scl = '1') then

state <= addr;

bit_count <= 7;

end if;

if (sda_d = '0' and sda = '1' and scl = '1') then

state <= idle;

sda_oe <= '0';

end if;

case state is

when idle =>

sda_oe <= '0';

when addr =>

if rising_edge(scl) then

rx_shift(bit_count) <= sda;

if bit_count = 0 then

rw <= sda;

state <= addr_ack;

else

bit_count <= bit_count-1;

end if;

end if;

when addr_ack =>

if rx_shift(7 downto 1) = slave_addr then

sda_oe <= '1';

bit_count <= 7;

if rw = '0' then

state <= reg_addr;

else

tx_shift <= mem(reg_ptr);

state <= read_data;

end if;

else

state <= idle;

end if;

when reg_addr =>

if rising_edge(scl) then

rx_shift(bit_count) <= sda;

if bit_count = 0 then

reg_ptr <= to_integer(unsigned(rx_shift));

state <= reg_ack;

else

bit_count <= bit_count-1;

end if;

end if;

when reg_ack =>

sda_oe <= '1';

bit_count <= 7;

state <= write_data;

when write_data =>

if rising_edge(scl) then

rx_shift(bit_count) <= sda;

if bit_count = 0 then

mem(reg_ptr) <= rx_shift;

state <= write_ack;

else

bit_count <= bit_count-1;

end if;

end if;

when write_ack =>

sda_oe <= '1';

state <= idle;

when read_data =>

if rising_edge(scl) then

sda_oe <= not tx_shift(bit_count);

if bit_count = 0 then

state <= read_ack;

else

bit_count <= bit_count-1;

end if;

end if;

when read_ack =>

sda_oe <= '0';

state <= idle;

end case;

end if;

end process;

end behavioral;

i2c top

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity i2c_top is

port(

clk,rst : in std_logic;

start : in std_logic;

rw : in std_logic;

addr7 : in std_logic_vector(6 downto 0);

reg_addr : in std_logic_vector(7 downto 0);

data_in : in std_logic_vector(7 downto 0);

busy : out std_logic;

ack_err : out std_logic;

done : out std_logic;

read_data : out std_logic_vector(7 downto 0)

);

end i2c_top;

architecture behavioral of i2c_top is

signal sda: std_logic := 'H';

signal scl : std_logic := 'H';

begin

master_inst : entity work.i2c_master

generic map(

i2c_freq => 100000,

clk_freq => 50000000

)

port map(

clk => clk,

rst => rst,

start => start,

rw => rw,

addr7 => addr7,

reg_addr => reg_addr,

data_in => data_in,

busy => busy,

done => done,

ack_err => ack_err,

read_data => read_data,

sda => sda,

scl => scl

);

slave_inst : entity work.i2c_slave

generic map(

slave_addr => "1000010"

)

port map(

clk => clk,

rst => rst,

scl => scl,

sda => sda

);

end behavioral;

i2c TestBench

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity i2c_tb is

end i2c_tb;

architecture behavioral of i2c_tb is

signal clk,rst : std_logic := '0';

signal start :  std_logic;

signal rw :  std_logic;

signal addr7 :  std_logic_vector(6 downto 0);

signal reg_addr :  std_logic_vector(7 downto 0);

signal data_in : std_logic_vector(7 downto 0);

signal busy :  std_logic;

signal ack_err :  std_logic;

signal done :  std_logic;

signal read_data :  std_logic_vector(7 downto 0);

begin

clk_process :

process begin

clk <= '0';

wait for 5 ns;

clk <= not clk;

wait for 5 ns;

end process;

top_inst : entity work.i2c_top port map(

clk => clk,

rst => rst,

start => start,

rw => rw,

addr7 => addr7,

reg_addr => reg_addr,

data_in => data_in,

busy => busy,

done => done,

ack_err => ack_err,

read_data => read_data

);

stim :

process begin

rst <= '0';

wait for 100 ns;

rst <= '1';

wait for 200 ns;

start <= '1';

rw <= '0';

wait for 40 ns;

start <= '0';

wait until done = '1';

wait for 500 ns;

start <= '1';

rw <= '0';

wait for 40 ns;

start <= '0';

wait until done = '1';

wait for  500 ns;

wait;

end process;

end behavioral;